[image: image1.png]




AB001- GMS RAM AIR KIT 98-01 YZF1000 R1

[image: image2.png]


[image: image3.png]Kol IAVAIE IV IVAON



[image: image4.png]


[image: image5.png]


[image: image6.png]


[image: image7.png]


[image: image8.png]Kol IAVAIE IV IVAON



[image: image9.png]


THIS PRODUCT IS FOR CLOSED CIRCUIT RACING ONLY AND SHOULD BE INSTALLED BY A QUALIFIED RACE TECHNICIAN.

[image: image10.png]



Remove the vacuum slide from the carburetor body. 


Remove the stock needle and spacers, noting the order of disassembly.


2.    Install the GMS needle clips on to the #3 clip position of the GMS needle. (Refer to figure # 1) The needle position is counted from the top. The small GMS washers are for half step increments and do not need to be installed at this time.


 Note: After installing slides in the carburetor be sure to check the slide movement before air box installation.


Install the main jets #146 provided in the kit. It is recommended to install the #146 main jets and increase or decrease the main jet size from there according to the demands of your motorcycle. 


Due to elevation and air density changes it may be necessary to install a smaller pilot jet. That needs to be determined by the race tech installing your kit. It is recommended to use the stock pilot jet #15 and only change the jet if a problem is noticed.


Locate the fuel mixture plug. (Refer to figure #2.) If you see a screw head, advance to the adjustment procedure. NOTE: The mixture screw is DIRECTLY beneath the brass plug so drill with caution. With the 5/32 drill bit provided in the kit carefully drill through the plugs, and be ready to pull back as soon as you break through. Use the screw provided to secure and remove the plug.        


Turn the fuel mixture screw clockwise and gently seat the screw then back out 3 turns.


6.     Locate the main air inlet at the back of the carburetor and insert 


the main air jet plug with the hollow end pointing out.


 NOTE: Make sure it is the main air jet that you are plugging.


7.     Remove the four rubber boots from the stock air box and insert them into the four large pre-cut holes in the GMS air box.


Route the float bowl vent tubes and lines strait up through the


Pre-drilled holes in the air box. Next cut the vent hose away leaving about ¾ of an inch left inside the air box. THIS IS CRUCIAL TO THE PERFORMANCE OF THE BIKE.


NOTE: The GMS airbox does not utilize a case vent. It is designed to run with a case vent catch can. Though not critical if you wish you can drill a hole into the GMS airbox for the case vent hose


NOTE: It is important for the airbox lid to have a good seal to the airbox bottom. To achieve this it might be necessary to sand down the high spots on either side of the box. 











� EMBED PBrush  ���





Figure #1


Put “E” clip on 3rd notch





� EMBED PBrush  ���





� EMBED PBrush  ���





Stock parts





GMS kit parts





Needle hole





� EMBED PBrush  ���





Figure #2


Fuel mixture screw





Contents		Qty





144 main jets		4


146 main jets		4


148 main jets		4


Main air plugs		4


GMS needles		4


GMS needle spacers	4


GMS “E” clips		4


GMS airbox		1


Installation instructions	1


5/32 drill bit		1


Plug removal screw	1


Air filter		1


Locking strip		1








� EMBED PBrush  ���





Main air jet





Main air jet








_1014532069

_1014535386

_1014611057

_1014532941

_1014526866

