<center><b><font color=green size=4><a name=”A2”>A.3. DSR Classification</a></font></b></center>

size 4 Times Bold

text:

<center><b><font color=green size=4><a name="A4">A.4. Safety Equipment</a></font></b></center>

<font size=2><p>Shall comply with GCR Section 17. In addition:</p>

<p>a. Glass headlight lenses and bulbs on the front of the car are prohibited.

<br> b. All Sports Racing Category cars shall provide protection for lower torso and legs of the driver by means of tubing and/or monocoque structure.

<br> c. Cars shall have two (2) red brake lights fitted with fifteen (15) watt (minimum) bulbs.

<br> d. Roll cages/roll bars shall comply with Section 18., for Sports Racers.</p>

<center><b><font color=green size=4><a name="A5">A.5. Bodywork</a></font></b></center>

<font size=2><p>Bodywork shall provide comfort and safety for driver and a passenger or for a driver only. All elements of the bodywork shall be completely and neatly designed and finished, with no temporary or makeshift elements.</p>

<p>a. The bodywork as viewed from the side and above shall cover all mechanical components except that the intake, exhaust, and radiators may be exposed. The bodywork shall extend over the full width of the tires for at least one third (1/3) of their circumference as viewed from the side. Ventilation slots are permitted. The tires shall not be seen as viewed from above, although the rear tires may be exposed as viewed from the rear. Cycle-type fenders (which only cover the tire and are not continuous with the rest of the body) are prohibited. Fenders shall be firmly attached to the bodywork with no gap between body and fender. Aerodynamic skirts are prohibited. See next Section for definition.</p>

<p>b. THIS SECTION APPLIES TO C & D SPORTS RACING ONLY:</p>

<p>It is the intent of these rules to minimize the use of "ground effects" to achieve aerodynamic downforce on the vehicle. Thus, for the full width of the body the floor pan will be a minimum of 45% of the wheelbase; the lower surface (surface licked by the airstream) shall not exceed 2.54cm (1 inch) deviation from the horizontal in any longitudinal section through that surface. (This is not to be interpreted as requiring a floor pan beneath the motor, transaxle, transmission, or final drive housing.) No aerodynamic devices (e.g. "skirts," body sides, etc.) may extend more than 1cm (0.394 inches) below this lower surface anywhere on the car to the rear of the front axle. Seat bucket or other protrusions shall not circumvent this rule. Aerodynamic devices shall be securely mounted on the entirely sprung part of the car and not be movable when the car is in motion. It is not permitted to duct air through any part of the bodywork for the purpose of providing aerodynamic downforce on the car. All ducted air for heat exchangers (water/oil) shall pass through these heat exchangers.</p>

<p>c. Dimensions</p>

<p>1. Height: No part of the vehicle having special or significant aerodynamic function shall exceed a height of 115cm (45.25 in) above the ground with car in normal racing trim, driver aboard. Neither the safety roll bar nor the engine induction intake shall provide an aerodynamic downforce.</p>

<p>2. Width: The maximum width shall not exceed 221 cm (87 inches) including all aerodynamic devices. However, no portion shall extend more than 10cm (3.9 inches) beyond a plane tangent to the outer face of the front and rear wheels with tires. The minimum body width between the front and rear wheels shall not extend inwards beyond a vertical plane connecting the centerlines of the front and rear tires.</p>

<p>3. Length: The maximum overall length shall be 485.3cm (191 inches).</p>

<p>4. Cockpit: The driver's seat shall be capable of being entered without the removal or manipulation of any part or panel (except for those closed cockpit cars which are specifically allowed by the SCCA). The cockpit opening shall comply with the following minimum dimensions for both single and two seater sports racers: Cockpit length: 60cm (23.662 inches) Cockpit width: 45cm (17.717 inches) maintained over 30cm (11.811 inches) from the most rearward point of the seat backrest toward the front.</p>

<p>d. Visibility: Bodywork shall provide visibility for the driver forward and to both sides adequate for racing conditions. Rear view mirror(s) shall provide the driver with visibility to the rear of both sides of the car.</p>

<p>e. Windscreens are optional.</p>

<p>f. Bodywork shall provide comfort and safety for both driver and a passenger. There shall be seats of equal dimension and comfort for the driver and a passenger equally disposed on each side of the longitudinal axis of the car. Note: Paragraph f. does not apply to single seat sports racers.</p>

<p>Seats shall be firmly attached in the car, but may provide adjustment for the size of the occupant. The body surrounding the driver and passenger compartment shall be symmetrical about the longitudinal axis of the car. The passenger's space and seat shall remain usable throughout the competition and shall not be encroached upon by an element of the car or equipment except as provided in these Rules.</p></font>

<center><b><font color=green size=4><a name="A6">A.6. Wheels and Tires</a></font></b></center>

<font size=2><p>There shall be no restriction on the size of wheels except for a minimum diameter of ten (10) inches, provided they are identical for the right and left front axles, and identical for the right and left rear axles. Left and right front tires will be the same size; left and right rear tires will be the same size.</p></font>

<center><b><font color=green size=4><a name="A7">A.7. Self Starter</a></font></b></center>

<font size=2><p>Cars shall be equipped with an automatic self starter and on-board power supply operated by the driver.</p></font>

<center><b><font color=green size=4><a name="A8">A.8. Brakes</a></font></b></center>

<font size=2><p>These cars shall be equipped with a dual braking system operated by a single control. In case of leak or failure at any point in the system, effective braking power shall be maintained on at least two (2) wheels. A separate hand brake (emergency brake) is not required.</p>

<center><b><font color=darkgreen size=4><a name="A"> A. SCCA Sports Racing Category Preparation Rules</a></font></b></center>

